Joseph Fiume

6133053

EE 215

Lab 7

10/30/2020

Objectives

Write a C-code program to use the LCD screen on the MSP430FR6989 board.

Explanation of Approach

To make use of the built in functions for the LCD screen, we have to import the “driverlib” folder. The files in driverlib must be included in the project. The LCD screen 1st displays my first name: JOSEPH for 2 seconds using the myLCD_showChar() function. Then it displays the numbers 1 through 6 in sequential order using the myLCD_displayNumber() function. The numbers are displayed in the rightmost location of the screen for one second each. Finally, the LCD displays the battery charging symbols using the myLCD_showSymbol() function. It cycles the battery charging symbols from the rightmost to the leftmost. After the battery is “full” (all the battery symbols are on), it blanks the screen for 1 second. This cycles infinitely.

Code

#include <msp430.h>

#include <driverlib.h> // Required for the LCD

#include "myGpio.h" // Required for the LCD

#include "myClocks.h" // Required for the LCD

#include "myLcd.h" // Required for the LCD

const unsigned int BATTERY[] =  //list of battery symbols

{

LCD_B6,

LCD_B5,

LCD_B4,

LCD_B3,

LCD_B2,

LCD_B1

};

void main(void)

{

WDTCTL = WDTPW | WDTHOLD; // Watchdog timer stop

// initialize LCD

initGPIO(); // Initializes inputs and outputs for LCD

initClocks(); // Initialize clocks for LCD

myLCD_init(); // Prepares LCD to receive commands

TA0CTL |= TASSEL_1|MC_1|ID_0; // ACLK, count up to CCR, divide clock by 1

TA0CCR0 = 0x8000;           // counter limit 0x8000 = 32768 = 1 second

unsigned int j = 1;             //create a flag for displaying the name

unsigned int i = 1;             //create an index for the numbers counting

int bat_index = 0;              //create an index for the battery symbols array

while(1)                        //loop forever

{

if (TA0CTL & TAIFG)   //If flag is set from counting up

{

if (j == 1)

{

myLCD_showChar( 'J', 1 ); // Display "J" in space 1

myLCD_showChar( 'O', 2 ); // Display "O" in space 2

myLCD_showChar( 'S', 3 ); // Display "S" in space 3

myLCD_showChar( 'E', 4 ); // Display "E" in space 4

myLCD_showChar( 'P', 5 ); // Display "P" in space 5

myLCD_showChar( 'H', 6 ); // Display "H" in space 6

j = 0;      //turn the name display flag off

}

else if (i <= 6)     //count from 1 to 6

{

myLCD_displayNumber(i);     //display the number on the LCD screen

i++;        //increment the number

}

else

{

myLCD_showChar( ' ', 6 );   //blank the LCD screen

if (bat_index <= 5)         //allow battery indexes in range of battery symbol list

{

myLCD_showSymbol(LCD_UPDATE, BATTERY[bat_index], 0);    //display the battery symbol at given index

bat_index++;        //increment the battery index

}

else

{

for (;bat_index >= 0; bat_index--)  //count down from the highest battery index 5-0

{

myLCD_showSymbol(LCD_CLEAR, BATTERY[bat_index], 0);    //blank the battery symbol at the given index

}

i = 1;      //set the numbers index back to its staring position

j = 1;      //set the name display flag

}

}

TA0CTL &= ~BIT0;    //Reset Flag

}

}

}

Results

The LCD screen displayed my name for 2 seconds. The LCD then displayed numbers from 1 to 6. The LCD screen then displayed the battery symbols cycling forever. The LCD loops forever.

Explanation of Results

The bits set to turn on LCD_B1 are Bit 5 in LCDM18. The memory register to turn on LCD_B1 is Pin 5.3 and COM1.

Conclusion

I learned how to use the built in functions to display letters, numbers and symbols on the LCD for the MSP430. I also learned how to include additional folders for a project in CCS.